Research visit proposal: microdosimetric validation of a computational neuroscience–radiobiology bridge

A request to connect an undergraduate thesis project (University of Pavia / CNAO) to the microdosimetric methodology developed at NIRS/QST, Chiba

Andrea Corti — University of Pavia, Department of Physics (TLB track)
September 2026

# 1. Purpose of this letter

I am a physics undergraduate at the University of Pavia (TLB track), currently preparing a thesis internship at CNAO (Centro Nazionale di Adroterapia Oncologica) for the coming year. Over the past months I have built, independently, an extended computational project connecting a spiking-neural-network model to a full Geant4-based simulation of CNAO's clinical proton and carbon-12 beams, down to nanodosimetric quantities and estimated DNA double-strand-break yields. The complete codebase and a 400+ page written account of the work (in Italian, with an English summary) are openly available on GitHub.

In building the radiobiological part of this project I relied repeatedly on the microdosimetric kinetic model (MKM) developed by Hawkins and refined for clinical use by Kase, Kanai and collaborators at NIRS — the same formalism, and in one case the same reference paper describing the TEPC-based biological-dose measurement used routinely for quality assurance at HIMAC. This letter is a request: I would like to spend part of my CNAO thesis period, or a dedicated visit if that is a more suitable arrangement, at NIRS/QST in Chiba, to validate and extend the weakest link in my own model using the instrumentation and data that only HIMAC can provide.

# 2. What the project does

The project has two connected halves. The first is a self-contained spiking-network study (Izhikevich neurons, Brian2): communication, synaptic plasticity, a ring attractor, and a learned bidirectional communication channel between two ring populations, extended to a three-population relay. The second half — the one directly relevant here — couples that network to a full physics chain built in Geant4 (v11.4.0, QBBC physics list for the clinical-beam simulation, Geant4-DNA for the nanoscale extension):

A ring of 100 tissue-equivalent spheres, positioned to match the network's own ring topology, is irradiated with CNAO's actual published clinical beam parameters (protons at 70/150/225 MeV, carbon-12 at 120/260/400 MeV/u), at both the entrance plateau and the true Bragg peak (located independently and validated within 1% against literature reference depths).

A functional "knockout" experiment silences the highest-dose neurons in the spiking network and shows a verified, direction-reversible, magnitude-graded interference effect on inter-population communication — confirmed across dozens of Monte Carlo seeds and four independent physical scenarios, and shown to generalize across knockout set sizes (a deliberately falsifiable direction-reversal prediction, confirmed).

A saturation-corrected MKM implementation (Hawkins/Kase, with the published HSG-cell parameters: α₀=0.13 Gy⁻¹, β=0.05 Gy⁻², domain radius rd=0.42 μm, saturation parameter y₀=150 keV/μm) computes RBE10 from the Geant4-simulated dose-averaged LET, validated before use against Kase et al.'s own published RBE10 values for a 290 MeV/u carbon-ion SOBP beam (within 10-15%, including at the clinical neutron-equivalent reference point, LET=80 keV/μm).

A tracking extension records the real energy spectrum of secondary electrons generated inside each irradiated volume (median ≈1.9 keV, 90th percentile ≈7.3 keV, maximum ≈96 keV — entirely within Geant4-DNA's validity range), and uses those real energies, rather than arbitrary monoenergetic points, as primaries in the official Geant4-DNA molecularDNA example to estimate single- and double-strand DNA break yields, spectrum-weighted across the full secondary-electron distribution.

Every quantitative claim in the project has been checked by actually running the code — including several cases where a first, surprising result turned out to be a genuine bug or a statistical artifact, diagnosed, fixed, and documented rather than hidden. I mention this because I expect it matters more to a research group than any single number in the report.

# 3. The specific limitation only HIMAC's instrumentation can address

The MKM implementation described above has one honestly-flagged, load-bearing simplification. The model formally requires the dose-mean lineal energy y_D, normally obtained from a measured tissue-equivalent proportional counter (TEPC) spectrum — the exact measurement described in Kase et al. (2011, J. Radiat. Res. 52:59-68), which reports TEPC-derived y* and RBE10 values for a 290 MeV/u carbon-ion SOBP beam at HIMAC. I do not have access to a TEPC or to measured lineal-energy spectra, so my implementation approximates y_D with the dose-averaged LET already computed by the Geant4 track simulation, and evaluates the saturation correction in its single-value (delta-spectrum) closed form rather than integrating over a full measured spectrum. The validation against Kase et al.'s published values (Section 2) shows this approximation is reasonable — within 10-15% — but I have no way, from Pavia, to test how much of that gap is the LET proxy itself versus other factors, or to extend the model to a properly spectrum-integrated saturation correction, because doing so requires exactly the kind of measured or simulated microdosimetric spectrum that HIMAC's TEPC infrastructure (and the group's accumulated experience with it) produces routinely.

# 4. What I would propose to work on

A concrete, appropriately-scoped piece of work for a visit of a few weeks to a month, building directly on the existing (open, documented, reproducible) codebase rather than starting from scratch:

Compare the LET-proxy MKM implementation against real or literature TEPC-measured y-spectra for beam configurations matching, as closely as possible, the CNAO clinical parameters already simulated — quantifying precisely where and why the proxy diverges from the full spectral calculation, rather than only bounding the gap as I currently can.

If feasible, replace the single-value saturation correction with the full spectral integral (Eq. 9 of the saturation-corrected MKM), using either measured HIMAC spectra or Geant4-DNA track-structure simulations of the relevant ion/energy combinations as a spectral input, and re-run the existing downstream chain (RBE-weighted neuron ranking, Sections 5.31–5.35 of the report) with the improved model to see whether any of the project's conclusions change.

More broadly, learn directly from the NIRS/QST group how the clinical microdosimetric measurement and calculation pipeline is actually run in practice — the step in this project I could only reconstruct from published papers, not from direct experience.

I am open to reshaping this around whatever the group considers most useful or feasible in the time available — the proposal above is a starting point, not a fixed plan.

# 5. Practical context

This request is tied to a thesis internship at CNAO planned for the next academic year; a period at NIRS/QST would ideally be arranged as part of, or adjacent to, that internship, in whatever way is administratively simplest on your side. I am glad to share the full codebase, the extended written report, and the specific code implementing the MKM model (with its validation check) referenced above, in whatever form is most convenient.

Repository (code, full report, and an English project summary): https://github.com/andreacorti88-spec/neural-simulation-project
Contact: andreacorti88@icloud.com